25 SEA COVE DRIVE

SketchUp Model → Permit Drawing Set Mapping

Model: "Seacove entry way concept"  |  Scope: Kitchen Remodel + Junior ADU  |  Date: April 2, 2026

# Permit Readiness Summary

Of the 19 sheets in the permit drawing set, your existing SketchUp model provides substantial content for 12 sheets. 4 sheets are fully ready to extract from the model, 8 need additional detail, and 7 require new content.

| Status | Count | Sheets | Action Required |
| --- | --- | --- | --- |
| READY | 4 sheets | A-1, A-4, A-7, A-9* | Extract views from model with minimal annotation |
| PARTIAL | 8 sheets | C-1, A-3, A-5, A-6, S-2, P-1, E-1, T24 | Model has geometry; needs dimensions, callouts, or additional elements |
| MISSING | 7 sheets | G-1, G-2, A-2, A-8, A-10, S-1, M-1 | New content required — either in model or as 2D drawings |

# Sheet-by-Sheet Mapping

How each permit sheet maps to your existing SketchUp model tags and what additional work is needed.

| Sheet | Title | Model Tags | Status | Notes / Work Needed |
| --- | --- | --- | --- | --- |
| G-1 | Cover Sheet / Title Block | — | MISSING | Title block with project info, vicinity map, sheet index |
| G-2 | General Notes & Abbreviations | — | MISSING | Code references, material specs, abbreviations |
| C-1 | Site Plan | Garden, Lawn, Pool | PARTIAL | Has landscape + pool. Needs property lines, setbacks, utilities |
| A-1 | Existing Floor Plan | Structure, bedroom, kitchen, Living Room, Dojo, Bonus Room | READY | Toggle off Proposed tags to show existing conditions |
| A-2 | Demolition Plan | kitchen (existing) | MISSING | Needs demolition hatching/notes on kitchen walls to remove |
| A-3 | Proposed Floor Plan | Structure + new JADU tag needed | PARTIAL | Entry concept modeled. Need formal JADU tag + dimensions |
| A-4 | Roof Plan | Roof, Solar panels, Solar roof | READY | Roof planes + solar already modeled. Add drainage arrows |
| A-5 | Building Sections (2) | Structure, Steel, wood, Roof | PARTIAL | Structure is there. Need wall cavity/assembly detail |
| A-6 | Exterior Elevations (4) | Structure, Roof, Steel, wood | PARTIAL | Geometry complete. Need material callouts + dimensions |
| A-7 | Interior Elevations - Kitchen | kitchen, CABINET, 3D-SINKS | READY | Cabinets + sinks modeled in detail |
| A-8 | Interior Elevations - JADU Bath | — | MISSING | JADU bathroom not yet designed/modeled |
| A-9 | Door & Window Schedule | Structure (openings) | PARTIAL | Openings exist. Need schedule table with types/sizes |
| A-10 | Finish Schedule | Interior - Furniture | MISSING | Room finish matrix needed |
| S-1 | Foundation Plan | — | MISSING | Slab/footings not modeled. Critical for JADU addition |
| S-2 | Framing Plan | Structure, Steel, wood | PARTIAL | Beams/posts visible. Need joist layout + connections |
| P-1 | Plumbing Plan | 3D-SINKS | PARTIAL | Fixtures placed. Need supply/waste piping layout |
| E-1 | Electrical Plan | Solar panels, Solar roof | PARTIAL | Solar modeled. Need circuits, panels, JADU sub-panel |
| M-1 | Mechanical Plan | — | MISSING | HVAC layout for JADU space needed |
| T24 | Title 24 Energy Compliance | Solar panels, Solar roof, Roof | PARTIAL | Solar + roof modeled. Calculations done separately |

# Existing Tag Inventory

Your model contains 18 tags. Here is how each one maps to the permit drawing set.

| Tag Name | Contains | Permit Sheets | How It Maps |
| --- | --- | --- | --- |
| 3D-SINKS | Kitchen/bath fixture models | P-1, A-7 | Plumbing fixture locations |
| bedroom | Bedroom wing rooms | A-1 | Existing floor plan |
| Bonus Room | Bonus/flex room | A-1 | Existing floor plan |
| CABINET | Kitchen cabinetry | A-7 | Kitchen interior elevations |
| Dojo | Exercise/martial arts room | A-1 | Existing floor plan |
| Fireplace | Fireplace element | A-1, A-5 | Floor plan + building section |
| Garden | Garden/planting areas | C-1 | Site plan landscape |
| Interior - Furniture | Interior furnishings | A-1 (ref only) | Not shown on permit drawings |
| kitchen | Kitchen room/elements | A-1, A-2, A-7 | Existing, demo, + interior elevations |
| Lawn | Lawn/turf areas | C-1 | Site plan landscape |
| Living Room | Living room area | A-1 | Existing floor plan |
| Pool | Swimming pool + deck | C-1 | Site plan |
| Roof | Roof planes | A-4, A-5, A-6 | Roof plan, sections, elevations |
| Solar panels | PV panel array | A-4, E-1, T24 | Roof plan, electrical, energy |
| Solar roof | Solar roof tiles/integration | A-4, E-1, T24 | Roof plan, electrical, energy |
| Steel | Steel structural members | S-2, A-5, A-6 | Framing, sections, elevations |
| Structure | Primary building structure | A-1 through A-6, S-2 | All architectural + structural sheets |
| wood | Wood structural/finish elements | S-2, A-5, A-6 | Framing, sections, elevations |

# New Tags to Add

These 8 new tags would complete the model organization for permit extraction. Add them in SketchUp before generating drawing set views.

| New Tag | What to Assign | Permit Sheets | Purpose |
| --- | --- | --- | --- |
| Proposed-JADU | New JADU addition geometry | A-3, A-5, A-6, A-8 | Isolate new construction from existing |
| Proposed-Kitchen | Kitchen remodel scope | A-2, A-3, A-7 | Separate demo from proposed work |
| Foundation | Slab + footings | S-1 | Below-grade structural elements |
| Plumbing | Supply + waste piping | P-1 | Plumbing rough-in layout |
| Electrical | Circuits + panels | E-1 | Electrical layout |
| Mechanical | HVAC ducts + equipment | M-1 | Mechanical layout |
| Property-Lines | Lot boundary + setbacks | C-1 | Required for RPV site plan |
| Demolition | Elements to be removed | A-2 | Demolition plan hatching |

# Recommended Workflow

Step 1: Organize (In SketchUp)

- Add the 8 new tags listed above to the model

- Assign the entry way concept geometry to "Proposed-JADU" tag

- Create existing-conditions Scenes by toggling off all Proposed tags

- Create proposed-conditions Scenes by toggling on all tags

- Add Section Cuts at key locations for building sections (A-5)

Step 2: Model New Elements (In SketchUp)

- JADU bathroom layout (fixtures, walls, door) → feeds A-8

- Foundation/footing geometry under JADU slab → feeds S-1

- Wall assembly detail (section cut through typical wall) → feeds A-5

- Property line rectangle with setback offset lines → feeds C-1

Step 3: Export to LayOut (For Permit Sheets)

- Create a LayOut file with 19 pages matching the sheet index

- Insert SketchUp model viewports using the Scenes created in Step 1

- Add dimensions, leaders, and annotations in LayOut (not SketchUp)

- Apply title blocks with project info, sheet numbers, and revision dates

- Export to PDF for RPV submission

Step 4: Consultant Coordination

- Structural engineer: Review S-1 and S-2 for beam/post calcs and foundation design

- MEP engineer: Produce P-1, E-1, M-1 from model fixture locations

- Energy consultant: Title 24 calculations using model roof/solar/window data

- Surveyor: Provide ALTA survey for C-1 property line accuracy